1. Basic Connectivity Tests

ping 192.168.1.1             	# Ping another device (IPv4)

ping 127.0.0.1               	# Ping loopback interface (IPv4)

ping ::1                    	# Ping loopback interface (IPv6)

ping www.google.com    # Ping domain (if DNS is configured)

2. Interface & IP Address Info

show ip interface brief     # Overview of interface status and IPs

show interface g0/1         	# Detailed info on GigabitEthernet0/1

show ipv6 interface brief   	# IPv6 interface overview

3. Access & Security Checks

show users                  	# See who is logged in

show line                   	# View line settings (console, VTY)

show ip ssh                 	# Check SSH status

4. Routing & Addressing

show ip route               	# IPv4 routing table

show ipv6 route             	# IPv6 routing table

show running-config | include ip address   # View all IP address configs

5. Device Info & Configs

show running-config        # View current configuration

show startup-config        # View saved startup config

show version                	# IOS version, system info

6. VLAN & MAC Table (Switch)

show vlan brief            	# Show all VLANs on the switch

show mac address-table      	# MAC address table

7. Path Testing (Routing)

traceroute 192.168.1.1      # Trace packet path to destination

8. Debug Commands (Advanced)

debug ip packet             	# Watch real-time IP packet processing

undebug all                 	# Turn off all debugging

Recommended Testing Sequence:

show ip interface brief

ping 192.168.1.1

traceroute 192.168.1.1

show running-config