LAN 1 (Router)

Router Performance Component: Configure Basic Settings

no ip domain-lookup

hostname Building-1

banner motd “ “

enable password ----

Router Configure Interfaces (g0/0/0, g0/0/1)

interface g0/0/0

description IT Department LAN

ip address ---.---.---.--- 255.255.255.---

no shutdown

----exit-----

interface g0/0/1

description IT Department LAN

ip address ---.---.---.--- 255.255.255.---

no shutdown

----exit-----

Router Configure Security Settings

enable secret ----

service password-encryption

security passwords min-length 10

Router Configure Interfaces and Secure Communication Lines (VTY and Console)

ip domain-name ------

username ---------- privilege 15 secret ----------

line console 0

password cisco12345

login

----exit----

line vty 0 4

password cisco12345

transport input ssh

login local

----exit----

ip ssh version 2

crypto key generate rsa general-keys modulus

1024

exit

copy running-config startup-config

LAN 2 (Switch)

Switch Performance Component: Configure Basic Settings

no ip domain-lookup

hostname Building-1

banner motd “ “

enable password ----

Switch Configure Interfaces (VLAN 1)

interface vlan 1

ip address ---.---.---.--- 255.255.255.---

no shutdown

ip default-gateway ---.---.---.---

Switch Configure Security Settings

enable secret ----

service password-encryption

Switch Configure Interfaces and Secure Communication Lines (VTY and Console)

ip domain-name -----

username ---------- privilege 15 secret ----------

line console 0

password cisco12345

login

----exit----

line vty 0 15

password cisco12345

transport input ssh

login local

----exit----

ip ssh version 2

crypto key generate rsa general-keys modulus

1024

Switch Secure and Shut Down Ports

Interface range ----, ----, ----, ---- (f0/1 – 4, f0/7 - 24, g0/1 - 2)

Description ------------

Shutdown

end

copy running-config startup-config